# AED Monthly Inspection Checklist

Automated External Defibrillator | FDA Class III Medical Device

| Date: ________________ | Location: ____________ | AED Serial #: ________ | Model: _______________ |
| --- | --- | --- | --- |
| Inspector: ___________ | Building/Floor: _____ | Battery Exp: _________ | Pad Exp: _____________ |

## Monthly Visual Inspection

| Inspection Item | Expected Condition | OK | Fail |
| --- | --- | --- | --- |
| Status Indicator | Green light or checkmark displayed. No warning symbols or red lights. |  |  |
| Battery Status | Battery installed and within expiration date. Charge indicator normal. |  |  |
| Adult Electrode Pads | Sealed in original packaging. Within expiration date. |  |  |
| Pediatric Pads | Present (if required) and within expiration date. |  |  |
| Physical Condition | No cracks, dents, or damage. Screen/display intact. Buttons undamaged. |  |  |
| Carrying Case | Clean, undamaged, opens/closes properly. |  |  |
| Cabinet/Mount | Cabinet undamaged, door opens freely. Alarm functional (if equipped). |  |  |
| Accessibility | AED visible, no obstructions. Location sign posted. |  |  |
| Temperature | Storage area within manufacturer range (typically 32°F–122°F). |  |  |
| Rescue Kit | CPR mask, razor, scissors, gloves, towel present and in date. |  |  |
| Self-Test Log | Device self-tests running. No error codes or alerts. |  |  |
| Signage | AED sign visible. Usage instructions posted. Emergency numbers displayed. |  |  |
| Manufacturer Alerts | No outstanding recalls or safety alerts for this model. |  |  |

## Annual Inspection Items (check if applicable this month)

| Inspection Item | Expected Condition | OK | N/A |
| --- | --- | --- | --- |
| Full Device Test | Full functional test per manufacturer instructions. Voice prompts clear. |  |  |
| Software/Firmware | Software current. Check manufacturer for updates. |  |  |
| Training Current | All designated responders have valid CPR/AED certification. |  |  |
| Replacement Planning | Battery and pad replacements ordered before expiration. |  |  |
| Program Documentation | Written AED program updated. Medical director oversight current. |  |  |
| Registration | Device registered with manufacturer for recalls and updates. |  |  |

## Deficiencies & Corrective Actions

| Deficiency Description | Corrective Action | Assigned To | Date Fixed |
| --- | --- | --- | --- |

AED Status:  READY for use /  OUT OF SERVICE – action required

Inspector Signature: ________________________________    Date: ______________

If any item fails inspection, take immediate corrective action. Never leave a non-functional AED in service. Contact the manufacturer for device issues. Document all actions taken.